# Summary of decisions relating to NSI architecture deliverable

This document is a central repository for recording decisions relating to the NSI architecture deliverable.

Version a1 updated 26 Oct 09

The following two letter codes are used to indicate contributors:

| Code | Person | Code | Person | Code | Person |
| --- | --- | --- | --- | --- | --- |
| AB | Athur Barczyk | JM | Joe Mambretti | YD | Yuri Demchenko |
| AL | Andy Lake | JV | John Vollbrecht |  |  |
| CD | Cees de Laat | ML | Mathieu Lemay |  |  |
| CG | Chin Gouk | MS | Martin Swany |  |  |
| EE | Eduard Escalona | NC | Nicola Ciulla |  |  |
| EG | Eduard Grasa Gras | NM | Nobutaka Matsumoto |  |  |
| EH | Evangelos Chaniotakis | PH | Petr Holub |  |  |
| FD | Freek Dijkstra | TL | Tom Lehman |  |  |
| GK | Gigi Karma-Edwards | RH | Richard Hughes-Jones |  |  |
| GR | Guy Roberts | RK | Radek Kryzwania |  |  |
| GZ | Georgios Zervas | RN | Ralph Niederberger |  |  |
| IM | Inder Monga | SS | Sebastien Soudan |  |  |
| JG | Joan A. Garcia-Espin | TM | Takahiro Miyamoto |  |  |
| JH | Jeroen van der Ham | TK | Tomohiro Kudoh |  |  |

| Issue No. | Date | Issue Short name | Issue Description | Comments | Decision/status |
| --- | --- | --- | --- | --- | --- |
| - 1 |  | Path vs Connection | Many names have been used to describe a path. These included path, circuit, lightpath, connection, label switched paths, switched virtual circuits etc. An appropriate name for the paths requests initiated using the NSI needs to be agreed. | GR The term ‘connection’ rather than circuit or path is proposed to differentiate between end-to-end paths which may typically terminate on an end-system and connections which may terminate on an intermediate-system i.e. organization switch. Also, ‘connection’ allows for multi-cast connections rather than just point-to-point. | Connection |
| - 2 |  | Connection oriented | Often paths or connections are classified as either connection-less or connection-oriented. Connection oriented circuits can be considered as circuits in which all packets take the same route. Will the NSI support request for connection-less paths? | GR Only requests for connection oriented circuits are within the scope of the NSI. | Only creation of connection-oriented type circuits will be supported by NSI |
| - 3 |  | Unidirectional/bidirectional | Typically in transmission technologies such as SDH or WDM only bidirectional connections are requested. In contrast, typically MPLS requires separate requests for each direction of transmission (Unidirectional). Which of these should the NSI support? | GR For consistency with GMPLS, bidirectional connection requests are proposed. Unidirectional requests to support multicast may be supported later? Unidirectional connections are described in NML |  |
| - 4 |  | Mulitpoint/ Multicast | Will NSI support point to multipoint connections and multicast? | Probably – how this is to be supported is still open |  |
| - 6 |  | Abstract vs. technology specific parameters | How are connection requests desctibed? | GR The mandatory parts of the service/connection request should include only technology no-specific attributes such as bandwidth (b/s) and globally unique identifiers. |  |
| - 7 |  | Technology specific attributes | How should technology specific attributes be incorporated in a NSI connection request? | Options proposed by FD: 1) Using the GMPLS approach of only a base model, and technology dependent extensions for each technology 2) Using the NML schema to make reference to technological details of ports 3) Using the Stitching Framework approach, by not only making the parameters flexible, but also making the logic flexible (of course that requires that beforehand, all possible logic needs to be known and handled by each control plane). GR 1: method 2 is proposed – if technology specific aspects need to be exchanged over the NSI these need to conform to NML. If technology specific aspects have only local significance the local NSA is free to use any technology specific naming scheme they wish. GR 2: Will there ever be a need to exchange technology specific attributes between NSAgents? Probably not, I suspect that it will nearly always be sufficient to exchange the globally unique id and find the tech attributes locally. |  |
| - 8 |  | NSI interface format | Should NSI nominate a preferred interface type for NSI (e.g. web services, XML), or leave this open? If yes, then which is preferred? | Under consideration |  |
| - 9 |  | Layers supported by NSI | On which layers should connections be able to be requested? | GR Connections should not be constrained to any particular layer – this is consistent with the abstraction requirement – Networks supporting all layers (1,2 and 3) will be free to implement an NSI interface. GR network edge ports may be found by the path-finding function, and the local network is free to nominate any layer deemed suitable. |  |
| - 10 |  | Connection transiting multiple technology domains | Will the NSI interface allow connections to be built across multiple technology domains? i.e multilple domains that may be managed by a single operator. | GR two options exist- - Each technology domains is managed a separate NSA - All technology domains maintained by a single operator are managed by a single NSA. Both situations are valid – it is up to the operator to ensure that the NSA is capable of controlling/managing the resources associated with that NSA. |  |
| - 11 |  | Multiple operator domains | Will the NSI interface support connection building over multiple operator domains? | GR Yes |  |
| - 12 |  | Heterogeneous networks | Will NSI support connections to be built across multiple domains with varying technology types? E.g a circuit transiting two domains one Ethernet and another SDH. This implies some ability to change technology specific attributes of the connection to suit the new technology. | GR Yes, support for connections across network of heterogeneous technologies is required. |  |
| - 13 |  | Segments | Should NSI support requests for only ete connections or should it also allow segments to be created which could later be concatenate to create an ete connection? | GR both segments and ete connections may be requested. |  |
| - 14 |  | User-to-network instance of NSI | Will a ‘North-bound’ instance of the NSI be supported? i.e. used for applications or middleware to request connections? | GR this is supported as an instance of the ‘recursive’ NSA model proposed by TK |  |
| - 15 |  | Network-to-network instance of NSI | Will a ‘horizontal’ instance of the NSI be supported? i.e. used for one network operator to forward connection requests to another network operator. | GR this is supported as an instance of the ‘recursive’ NSA model proposed by TK |  |
| - 16 |  | Naming of circuit end-points | How will connection/segment end points be named? | JV Connections source and connection destination naming will follow NML recommendations. In accordance with NML, each connection end points will have an identifier which is globally unique. GR This may include both a physical port component and a logical port component. |  |
| - 17 |  | Naming connections | How should connections be named? | JV ete connections will have a globally unique name GR, I suggest a globally unique name for ete connections only, and any segments can be defined as pairs of ports. |  |
| - 18 |  | Naming domains | How should NSA be named? | GR Each NSA should have a globally unique name to allow NSA finding. |  |
| - 19 |  | Friendly end-point names | How can human friendly names be found for connection end points? | GR this may be provided by an information service GR note that 3 types of name are implied: - Technology specific that can be used to configure data plane - Globaly unique id that can be exchanged between NSAs - User friendly name that has human meaning |  |
| - 20 |  | Topology sharing | How can topology be shared between domains? | JV An NSA should be able to share its aggregated data-plane topology with middleware and other NSAs. Topology is shared in either a globally unique name or directly in NML format, and consists of a set of networks, network edge ports and links. GR There needs to be an association between an NSA and its resources – this will allow the ‘NSA finding’ service to know which NSA to go to, to request a segment. This requirement should be accommodated in the NML naming scheme? |  |
| - 21 |  | Connection definition attributes | Which attributes can be specified when requesting a path | JM: Directionality [Uni] [Bi] [Multi] Priority [Max] [Min] Redundancy [Max] [Min] Reliability [Max] [Min] Security [Type] Reserved [N1] [N2] …[Nn] |  |
| - 22 |  | Connection performance parameters | Which technology abstracted parameters for the specification of the connection performance should be supported? | GR Throughput (NA for best effort?) Jitter MTU Latency JM: Capacity [Max] [Min] Quality Attribute 1 [Max] [Min] |  |
| - 23 |  | Advance reservation | Is advance reservation of connections a requirement? | GR Yes |  |
| - 24 |  | Connection end-point types | What connection end-points are within scope? i.e what layer should end-point reach. | JS The following describes several types of use cases: - Request to connect to/from a facility (eg Ethernet switch on campus) - Request to connect to/from a host (could be either Ethernet port on switch facing host, or Ethernet port on host itself) - Request to connect to/from an application (TCP port and/or socket) |  |
| - 25 |  | Proxy requests | Does the request need to originate from the end-system or are proxy requests supported? | JS The “proxy” example of a use case is the dumb device (for example a video camera) that does not even know the wire emerging from the device is attached to the network. A user proxy agent somewhere makes a service request of the network to connect the switch port to which the camera is attached to some other end point across town. |  |
| - 26 |  | Routing | Does the NSI interface need to support any routing information? | GR It is not mandatory for a request to include routing information. Some requests may be network aware and wish to specify some routing parameters, such as partial or fully completed routes. |  |
| - 27 |  | User feedback | If for some reason a path request fails, should the NSI interface support feedback to the user/middleware? How much information should be returned? | GR – Yes, details to be determined |  |
| - 28 |  | Autonomous domains/networks | Autonomy and local policy | GR Each domain holds its own reservation calendar. The domain can accept or reject any local connection request for any reason. |  |
| - 31 | 15 Oct 09 | Synchronous vs. asynchronous | Should the client always initiate a request response message interaction? i.e will client always need to initiate a request – this is synchronous | IM – no way of solving problem with out using asynchronous solution. GR Preemption requires asynchronous usage. Could make asynchronous optional. I.e. basic message set is synchronous and extensions require asynchronous additions. |  |
| - 32 | 15 Oct 09 | Reuse of standards eg RSVP | Should NSI interface be able to implement RSVP? | IM – new TLVs to extend RSVP by basic set of existing RSVP message set is re-used |  |
| - 33 | 15 Oct 09 | How is protocol spec split? | See doc OIF UNI-2.0 RSVP extensions as an example of how this can be done. | IM - should have two parts- NSI common messaging (includes?) NSI transport stack, for example html, xml, soap |  |
| - 33 | 15 Oct 09 | Interface naming | How to name two instances of NSI? | Suggestion: NSI/A (agent) and NSI/N (network) Is there a difference? Perhaps these should not be highlighted –common messaging on both, but not ‘agent finding’ |  |
| - 34 | 15 Oct 09 | Path-finding naming | Path-finding is a poor name for NSA path finding What would be a better name? | ‘Agent finding’ service could be better. |  |
| - 35 | 15 Oct 09 | Topology exchange naming | Topology exchange is a poor name for NSA topology exchange. What would be a better name? | ‘NSA topology exchange’ |  |
| - 35 | 15 Oct 09 | Data plane naming | Data plane vs. transport plane? |  |  |
| - 36 | 15 Oct 09 | OGF27 session1 Can link and network concepts be merged into a single concept? | IM asked if the concept of ‘transport resource link’ is sufficiently different to a 2 port network to require its own definition. | Inder thinks that these are sufficiently similar to not require a separate link concept. |  |
| - 37 | 15 Oct 09 | Forwarding ports definition? | Definition is agreed – just needs clear definition. Martin S. suggested example similar names eg forwarding port = logical port | Forwarding ports are defined as the ‘logical ports’ within a physical port |  |
| - 38 | 15 Oct 09 | Access port definition | What is the definition of an ‘Access port’ | Port at the end-point of an ete connection |  |
| - 39 | 15 Oct 09 | Routing areas definition | Networks can be aggregated into routing areas. | These are areas which have full reachability. If end address is outside of routing area, the area will have a forwarding table to appropriate routing area. |  |
| - 40 | 15 Oct 09 | Service plane topology | Terms – requestor agent NS Agent Local resource Recursive resource | Terms need to be reviewed and agreed |  |
| - 41 | 15 Oct 09 | Network resource – better definition | Suggest: network service resources Network service provider Network resource service Service resource network Network provider Network service |  |  |
| - 42 | 15 Oct 09 | Requestor agent - Better definition | TK suggests that we identify sub parts of NSA as: -NSA client NSA server |  |  |
| - 43 | 15 Oct 09 | Transport terminology | Are definitions and names ok for: Network (same as sub-network in ITU-T G.800) Link Port (networks connect here) Access ports (users connect) Segment (may be recursively concatenated) Forwarding port (logical port in port) Segment concatenation Network connection (between access ports) Topology Routing area | Some definition issues: Should link be replaced with 2 port network? Port – currently belongs to network only – should this be assigned to ends of a links? JV – these form the basis of a set of requirements for NML-WG to formalize. |  |
| - 44 | 15 Oct 09 | Definition of segment/connection/path not clear | Segments may be concatenated to form longer segments JH - do we need to have a separate name for an instantiated path. | MS suggests that the connection is an instantiated version of a path |  |
| - 45 | 15 Oct 09 | Routing area naming | Is this the best naming – overloaded? | GR – ‘routing area’ is loaded with too many IP connotations. |  |
| - 46 | 15 Oct 09 | Topology collection | How does the NSA collect the aggregated data plane topology? | The details of how topology sharing is done and PCE are out of scope of this NSI standard. GR - How this is done is out of scope of NSI, but must support: Each domain must be able to advertise local connectivity. This should include information on which networks are at far-end of link. A service must exist which accept these local connectivity advertising. Also, this service must be able to respond to a NSA request for network connectivity. JV –NS service finding is out of scope of NSI – but this service must exist. |  |
| - 47 | 15 Oct 09 | Naming for ‘NS Agent finding’ | What name should be used for ‘NS Agent finding’ | GR – I don’t like this name. It implies that the service tells only how to reach a particular NSA. Actually the information is more detailed. Service should answer the question – which NSA to I go to be able to forward my transport plane connection? I.e this is the functionality required to support the ‘Recursive resource’. |  |
| - 48 | 15 Oct 09 | Path | JV – this is a routing concept and is not a connection. So a ‘path’ is returned by the PCE. | GR if ‘path’ is chosen to mean a set of routing ports, this needs to be clearly written down – to |  |
| - 49 | 15 Oct 09 | Required services | GR Two distinct service are required – ‘path computation service’ ‘agent forwarding service’ |  |  |
| - 50 | 15 Oct 09 | Exclusion of resources | Should allow connection request be allowed to explicitly exclude specific resources? | GR - yes |  |
| - 50 |  |  |  |  |  |
| - 50 |  |  |  |  |  |