Anycasting and the Network Service Interface

Joan A. García-Espín (i2CAT)

Contents:

Anycasting	1

Examples	2

Simple workflow (I) – figure 2	3

Simple workflow (II) – figure 3	4

Effects of the Anycast support on the NSI	5

Acronyms	6

References	6

## Anycasting

Anycast is a type of data transmission where data is sent from single source to the ‘nearest’ or ‘best’ destination point [1]. Note that both ‘nearest’ and ‘best’ adjectives need to be contextualised (network cost, Geo-spatial coordinates, application constraints, or others).

The group of receivers is identified with single routing address, a.k.a. Anycast address, however only one is receiving the data at the same time (cf. Figure 1).

1

2

3

4

Figure 1: Anycasting (source: [1]).

In Figure 1, yellow circle is the sender of the service request (Requestor Agent), while blue circles –numbered 1 to 4- are the feasible receivers of the request (NS Agents). RA will send its request to any NSA in the service plane without specifying the destination end-point, but the service or non-network resource needed. Once evaluated in the NSA, the Anycast NS request will initiate a workflow within the service plane in order fetch the end-point associated to the service requested and reserve all the needed network resources along the computed path.

It must be said Grid environments already commonly use point-to-multipoint connections but multicasting and anycasting are more experimental ones and don’t typically meet in operational computation infrastructures [1].

## Examples

- Large data transfers/storage.

This is a typical use case present in many professional areas: Grid-FTP-like applications, LHC’s data set collection jobs, enterprise’s data back-ups, storage services ala Cloud, data centres life-motif, and a long etc.

- Remote processing workflows.

Major Industry and Academia entities typically have their own agreements with (super-) computing centres (or even their own computing facilities). However, there is a big pond of smaller centres that cannot afford it and would occasionally like to take profit of that (remote) processing power for their own workflows/purposes. Basing on anycasting service, small centres could occasionally ask for the processing power required and see if it fits their budget, independently of the location of the computing facilities (assuming no large data sets are produced).

- Media services: streaming

Peer-to-peer (p2p) networks allow a kind of anycasting service (note that files downloaded are not only located in one server, but spread over several peers in small parts). However, live streaming of (HD, super HD, 4k, 8k, …) media is not feasible with today’s p2p solutions. Real anycasting service, as described in [2], could fulfil this purpose.

- Pervasive IT services (cf. [5])

IaaS (IT definition) and SaaS trends in Future Internet are making internal company services to request for pervasive servers. It must also be noted that some companies even externalise them (i.e. data storage). Moreover, users tend to use wireless access to IT services, which in many cases causes moving service access points to several locations in the network (i.e. distributed security systems in large campuses or companies).

## Simple workflow (I) – figure 2

Figure 2: Simple Anycasting: NSA1 has the mapping ‘service’-‘EP’.

Workflow description (created using [3]):

RA->NSA1: NS_Req(srcEP, service)

activate NSA1

NSA1->NSA1: look for 'service' \n in stored topo info \n and get 'dstEP'

note left of NSA1

NSA1 has a table

with mapping

'service'-'EP'

end note

note left of NSA1

('service','EP') tuple

advertisement

is needed among NSAs

end note

NSA1->NSA1: compute path \n -> route

NSA1->NSA2: NS_Req(route)

deactivate NSA1

activate NSA2

opt

note over RA,NSA1: If NMS/CP supporting resource scheduling calls

NSA2->NMS/CP: Availability_Poll

NMS/CP->NSA2: Resource_Status

end

NSA2->NSA1: NS_Resp(resvID)

deactivate NSA2

activate NSA1

NSA1->RA: NS_Resp(resvID, [dstEP])

deactivate NSA1

## Simple workflow (II) – figure 3

Figure 3: Simple Anycasting: external entity to map ‘service’ to ‘EP’ (DNS-like).

Workflow description (created using [3]):

RA->NSA1: NS_Req(srcEP, service)

activate NSA1

NSA1->NSA0: Fetch(service)

deactivate NSA1

activate NSA0

NSA0->NSA1: Location(service, EP)

deactivate NSA0

activate NSA1

note over NSA0

('service','EP') tuple

advertisement

is needed among NSA

in charge of mapping

end note

NSA1->NSA1: compute path \n -> route

NSA1->NSA2: NS_Req(route)

deactivate NSA1

activate NSA2

opt

note over RA,NSA1: If NMS/CP supporting resource scheduling calls

NSA2->NMS/CP: Availability_Poll

NMS/CP->NSA2: Resource_Status

end

NSA2->NSA1: NS_Resp(resvID)

deactivate NSA2

activate NSA1

NSA1->RA: NS_Resp(resvID, [dstEP])

deactivate NSA1

## Effects of the Anycast support on the NSI

Several considerations regarding network/non-network resources and their availability in the interface must be taken into account. Among them, it is very critical to consider:

- Destination EP is not explicitly specified in the service request

- Destination EP is sorted out from service/non-network resource requested (implicit destination)

- NSAs should make public the services they can fetch or register them in a centralised, DNS-like resolution system.

- NSAs need to dialogue with non-network systems, in order to fetch their available services (part of the NSI?)

- The level of detail in service description influences destination end-point selection process (best-match typically used)

- A new set of non-network resource description is needed. In the case of Grid services, description of site, service, cluster, computing/storage element/area, software or host is needed [4].

## Acronyms

CP	Control Plane

EP	End-point

HD	High Definition

LHC	Large Hadron Collider

NMS	Network Management System

NS	Network Service

NSA	Network Service Agent

RA	Requestor Agent

## References

[1]	N. CIULLI et al., The Grid-GMPLS Control Plane Architecture, Phosphorus deliverable D2.1, the Phosphorus Project (European contract no. 034115), March 2007.

[2]	C. Partridge, T. Mendez, W. Milliken, Host Anycasting Service, RFC 1546, IETF, November 1993.

[3]	Web Sequence Diagrams, www.websequencediagrams.com

[4]	N. CIULLI et al., Routing and Signalling Extensions for the Grid-GMPLS Control Plane, Phosphorus deliverable D2.2, the Phosphorus Project (European contract no. 034115), September 2007.

[5]	J. ABLEY, K. LINDQVIST, Operation of Anycast Services, RFC 4786, IETF, December 2006.

Service plane must be understood as a federation of NSAs where a given service request to one entity can be served by the same entity or be forwarded to a more suitable NSA. A service plane can either be a control plane (i.e. GMPLS), a network Management system or a network resource provisioning system.