# Scenario analysis for connection requests:

This document is intended to be a discussion document for considering an appropriate set of parameters for connection requests.

A set of strawman parameters are proposed and used as the basis for a set of scenarios.

Request-id – globally unique id for the request

Request type – reservation, instantiation, cancel, poll --

ReservationId – Globally unique reservation id

SignallingType – Signalled or on-demand instantiation

SrcNsa – id of therequesting NSA

DestNsa – id of the serving NSA

StartTime – reservation start time

EndTime – reservation end time

Bandwidth – reservation bandwidth

PathRouting (StartPort, EndPort, Segments) - Path routing details

StartPort – the connection starting point port address – should be in conformance with NML schema

EndPort – the connection starting point port address – should be in conformance with NML schema

Segments – pair of ports describing a routing segment

# Connection request scenarios.

This arbitrary set of scenarios has been created as a basis for analysing the way in which a connection request is propagated.

## Scenario 1: request to a single NSA

Network A

A1

A2

A3

NSA

A

NSA

Z

NSI

Figure 1: -Single network request

ResId

Res1SrcNsa

NSA_ZDestNsa

NSA_AStartPortA1EndPortA2Segment1A1, A2Segment2-Segment3-

Segment3-

## Scenario 2: request to 2 NSAs - chain type

B1

Network A

Network B

B2

B3

B4

A1

A2

A3

NSA

A

NSA

B

NSA

Z

NSI

NSI

ResId

Res2SrcNsa

NSA_ZDestNsa

NSA_AStartPortA1EndPortB3Segment1A1, A2Segment2A2, B1Segment3B1, B3

Segment4-

Figure 2: request to 2 NSAs - chain type

ResId

Res2SrcNsa

NSA_ADestNsa

NSA_BStartPortB1EndPortB3Segment1B1, B3

Segment2-Segment3-

Segment4-

[I think the implication of this is that NSA A has an RMA that can allocate Network A as well as the link a2-B1.  Perhaps this should be stated?]

A question is how does Z represent the endpoints as well as the segments.    This seems quite possible requirement.  Alternatively NSA A could receive the request for endpoints only and break the request locally.  In a GMPLS type chain it would be forward the  xegments of the path satisfied by A as well as the segments required from B.

## Scenario 3: request to 2 NSAs - tree type

B1

Network A

Network B

B2

B3

B4

A1

A2

A3

NSA

A

NSA

B

NSA

Z

NSI

NSI

ResId

Res2SrcNsa

NSA_ZDestNsa

NSA_AStartPortA1EndPortA2Segment1A1, A2Segment2-Segment3-Segment4-

Figure 3: request to 2 NSAs - tree type

ResId

Res2SrcNsa

NSA_ZDestNsa

NSA_BStartPortB1EndPortB3Segment1B1, B3

Segment2-Segment3-

Segment4-

Who reserves the A2-B1 segment?

## Scenario 3: request to 3 NSAs - chain type

B1

Network A

Network B

Network D

Network C

B2

B3

B4

A1

A2

A3

D1

D2

D3

D4

D5

C1

C2

C3

C4

C5

NSA

A

NSA

B

NSA

C

NSA

D

NSA

Z

NSI

ResId

Res2SrcNsa

NSA_ZDestNsa

NSA_BStartPortB1EndPortD4Segment1B1, B3Segment2B3, C1Segment3C1, C5

Segment4C5, D2Segment5D2, D4

NSI

ResId

Res2SrcNsa

NSA_BDestNsa

NSA_CStartPortC1EndPortD4Segment1C1, C5Segment2C5, D2Segment3D2, D4

Segment4-Segment5-

ResId

Res2SrcNsa

NSA_CDestNsa

NSA_DStartPortD2EndPortD4Segment1D2, D4Segment2-Segment3-Segment4-Segment5-

NSI

NSI

Figure 4: request to 3 NSAs - chain type

I am confused about why A is present in this pic.   Perhaps to show connectivity of NSAs?

## Scenario 4: request to 4 NSAs - hybrid type

B1

Network A

Network B

Network D

Network C

B2

B3

B4

A1

A2

A3

D1

D2

D3

D4

D5

C1

C2

C3

C4

C5

NSA

A

NSA

B

NSA

C

NSA

D

NSA

Z

NSI

ResId

Res2SrcNsa

NSA_ZDestNsa

NSA_AStartPortB1EndPortD4Segment1B1, B3Segment2B3, C1Segment3C1, C5

Segment4C5, D2Segment5D2, D4

ResId

Res2SrcNsa

NSA_ADestNsa

NSA_BStartPortB1EndPortC5Segment1B1, B3Segment2B3, C1Segment3C1, C5

Segment4-Segment5-

NSI

ResId

Res2SrcNsa

NSA_BDestNsa

NSA_CStartPortC1EndPortC5Segment1C1, C5

Segment2-Segment3-Segment4-Segment5-

ResId

Res2SrcNsa

NSA_ADestNsa

NSA_DStartPortD2EndPortD4Segment1D2, D4Segment2-Segment3-Segment4-Segment5-

NSI

NSI

Figure 5: request to 4 NSAs – hybrid type