3GPP TS 29.199-2 V8.0.0 (2007-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

Open Service Access (OSA);

Parlay X Web Services;

Part 2: Third party call

(Release 8)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

UMTS, network, API, OSA

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2007, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	7

4	Detailed service description	7

5	Namespaces	9

6	Sequence diagrams	9

6.1	'Click to Dial' call setup	9

6.2	Third party call with announcement between broker and customer	10

7	XML Schema data type definition	10

8	Web Service interface definition	11

8.1	Interface: ThirdPartyCall	11

8.1.1	Operation: MakeCallSession	11

8.1.1.1	Input message: MakeCallSessionRequest	11

8.1.1.2	Output message : MakeCallSessionResponse	12

8.1.1.3	Referenced faults	12

8.1.1a	Operation: AddCallParticipant	12

8.1.1a.1	Input Message : AddCallParticipantRequest	12

8.1.1a.2	Output message : AddCallParticipantResponse	12

8.1.1a.3	Referenced faults	12

8.1.1b	Operation: TransferCallParticipant	13

8.1.1b.1	Input Message : TransferCallParticipantRequest	13

8.1.1b.2	Output message : TransferCallParticipantResponse	13

8.1.1b.3	Referenced faults	13

8.1.2	Operation: GetCallParticipantInformation	14

8.1.2.1	Input message: GetCallParticipantInformationRequest	14

8.1.2.2	Output message : GetCallParticipantInformationResponse	14

8.1.2.3	Referenced faults	14

8.1.3	Void	14

8.1.4	Void	14

8.1.5	Operation: GetCallSessionInformation	14

8.1.5.1	Input message: GetCallSessionInformationRequest	14

8.1.5.2	Output message : GetCallSessionInformationResponse	15

8.1.5.3	Referenced faults	15

8.1.6	Operation: DeleteCallParticipant	15

8.1.6.1	Input message: DeleteCallParticipantRequest	15

8.1.6.2	Output message: DeleteCallParticipantResponse	15

8.1.6.3	Referenced faults	15

8.1.7	Operation: EndCallSession	16

8.1.7.1	Input message: EndCallSessionRequest	16

8.1.7.2	Output message: EndCallSessionResponse	16

8.1.7.3	Referenced faults	16

9	Fault definitions	17

9.1	ServiceException	17

9.1.1	SVC0260: Void	17

9.1.2	SVC0261: Call Session already terminated	17

10	Service policies	17

Annex A (normative):	WSDL for Third Party Call	18

Annex B (informative):	 Description of Parlay X Web Services Part 2: Third party call for 3GPP2 cdma2000 networks	19

B.1	General Exceptions	19

B.2	Specific Exceptions	19

B.2.1	Clause 1: Scope	19

B.2.2	Clause 2: References	19

B.2.3	Clause 3: Definitions and abbreviations	19

B.2.4	Clause 4: Detailed service description	19

B.2.5	Clause 5: Namespaces	19

B.2.6	Clause 6: Sequence diagrams	20

B.2.7	Clause 7: XML Schema data type definition	20

B.2.8	Clause 8: Web Service interface definition	20

B.2.9	Clause 9: Fault definitions	20

B.2.10	Clause 10: Service policies	20

B.2.11	Annex A (normative): WSDL for Third Party Call	20

Annex C (informative):	Change history	21

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

3GPP acknowledges the contribution of the Parlay X Web Services specifications from The Parlay Group. The Parlay Group is pleased to see 3GPP acknowledge and publish the present document, and the Parlay Group looks forward to working with the 3GPP community to improve future versions of the present document.

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part 2 of a multi-part deliverable covering the 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Open Service Access (OSA); Parlay X Web Services, as identified below:

Part 1:	"Common"

Part 2:	"Third party call"

Part 3:	"Call Notification"

Part 4:	"Short Messaging"

Part 5:	"Multimedia Messaging"

Part 6:	"Payment"

Part 7:	"Account management"

Part 8:	"Terminal Status"

Part 9:	"Terminal location"

Part 10:	"Call handling"

Part 11:	"Audio call"

Part 12:	"Multimedia conference"

Part 13:	"Address list management"

Part 14:	"Presence"

Part 15:	"Message Broadcast"

Part 16:	"Geocoding"

Part 17:	"Application driven Quality of Service (QoS)"

Part 18:	"Device Capabilities and Configuration"

Part 19:	"Multimedia streaming control"

Part 20:	"Multimedia multicast session management"

# 1	Scope

The present document is Part 2 of the Stage 3 Parlay X Web Services specification for Open Service Access (OSA).

The OSA specifications define an architecture that enables application developers to make use of network functionality through an open standardized interface, i.e. the OSA APIs. The concepts and the functional architecture for the OSA are contained in 3GPP TS 23.127 [3]. The requirements for OSA are contained in 3GPP TS 22.127 [2].

The present document specifies the Third Party Call Web Service aspects of the interface. All aspects of the Third Party Call Web Service are defined here, these being:

- Name spaces.

- Sequence diagrams.

- Data definitions.

- Interface specification plus detailed method descriptions.

- Fault definitions.

- Service Policies.

- WSDL Description of the interfaces.

The present document has been defined jointly between 3GPP TSG CT WG5, ETSI TISPAN and The Parlay Group.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.127: "Service Requirement for the Open Services Access (OSA); Stage 1".

[3]	3GPP TS 23.127: "Virtual Home Environment (VHE) / Open Service Access (OSA); Stage 2".

[4]	3GPP TS 22.101: "Service aspects; Service principles".

[5]	W3C Recommendation (2 May 2001): "XML Schema Part 2: Datatypes". http://www.w3.org/TR/2001/REC-xmlschema-2-20010502/.

[6]	3GPP TS 29.199-1: "Open Service Access (OSA); Parlay X Web Services; Part 1: Common".

[7]	3GPP TS 29.199-12: "Open Service Access (OSA); Parlay X Web Services; Part 12: Multimedia conference”.

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TS 29.199-1 [6] apply.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TS 29.199-1 [6] apply.

# 4	Detailed service description

Currently, in order to perform a third party call in telecommunication networks we have to write applications using specific protocols to access Call Control functions provided by network elements (specifically operations to initiate a call from applications). This approach requires a high degree of network expertise. We can also use the OSA gateway approach, invoking standard interfaces to gain access to call control capabilities, but these interfaces are usually perceived to be quite complex by application IT developers. Developers must have advanced telecommunication skills to use Call Control OSA interfaces.

In this subclause we describe a Parlay X Web Service, Third Party Call, for creating and managing a call initiated by an application (third party call). The overall scope of this Web Service is to provide functions to application developers to create a call in a simple way. Using the Third Party Call Web Service, application developers can invoke call handling functions without detailed telecommunication knowledge.

The underlying model of the service is based on the following entities:

- Call Session: a call (uniquely identified) to which participants can be added/removed.

- Call Participant: each of the call parties (uniquely identified) involved in the call session.

- Media: the call can utilize multiple media types to support the participants' communication. In particular both audio and video streams are available, including the specific stream direction (i.e. incoming, outgoing, bidirectional).

NOTE 1:	Call participants in a Call Session are anticipated to be uniquely identifiable using their URI address.

An application setting up a call session must initially invoke the makeCallSession. The result of such invocation is the creation of a "context" that represents a call session with usually two participants, or at a minimum one participant connected, a unique identifier is assigned to the just-created call session. Subsequently the application may wish to add, remove, park or transfer call participants. In order to do so the operations addCallParticipant, transferCallParticipant, deleteCallParticipant can be used. Furthermore the call session or call participant status including the media details can be read. In order to do so the operations getCallParticipantInformation, and getCallSessionInformation can be used. It is also possible to retrieve the media details on its own using the getMediaForParticipant or getMediaForCall operations of the Audio Call web service.

The application can also force the call session and all its participants to be terminated with the operation endCallSession.

NOTE 2:	A call session allows the application to avail of other web service features that can add value to the created call session. For example the Audio Call web service can provide multimedia message delivery to call participants in the call session (playXXXMessage operation) and furthermore control of the media types for the call participants thus enabling conversational multimedia communication including voice, video, chat, and data. Media can be added/removed for each participant.

Figure 4.1 shows a scenario using the Third Party Call Web Service to handle third party call functions. The application invokes a Web Service to retrieve stock quotes and a Parlay X Interface to initiate a third party call between a broker and his client.

In the scenario, whenever a particular stock quote reaches a threshold value (1) and (2), the client application invokes a third party call between one or more brokers and their corresponding customers to decide actions to be taken. After invocation (3) by the application, the Third Party Call Web Service invokes a Parlay API operation (4) using the Parlay/OSA SCS-CC (Call control) interface. This SCS handles the invocation and sends a message (5) to an MSC to set-up a call between user A and user B.

In an alternative scenario, the Parlay API interaction involving steps (4) and (5) could be replaced with a direct  interaction between the Third Party Call Web Service and the Mobile network

Figure 4.1: Third party call scenario

# 5	Namespaces

The ThirdPartyCall interface uses the namespace:

http://www.csapi.org/wsdl/parlayx/third_party_call/v3_4

The 'xsd' namespace is used in the present document to refer to the XML Schema data types defined in XML Schema [5]. The use of the name 'xsd' is not semantically significant.

# 6	Sequence diagrams

## 6.1	'Click to Dial' call setup

A common convergence application is Click to Dial, where a self service portal provides a web page that can initiate a call between two phones. This sequence shows a basic call setup, and ending the call through the portal.

## 6.2	Third party call with announcement between broker and customer

This sequence shows the third party call with announcement between broker and customer in addition to basic flow of figure 4.1. It could be realized through invoking TPC and AC web services with same session identifier. If the broker gets the announcement of information related to this call, he can get more information before talking to his customer.

# 7	XML Schema data type definition

Void.

# 8	Web Service interface definition

## 8.1	Interface: ThirdPartyCall

This interface provides the ability to setup a call session, add and delete a call participant, transfer a call participant from one call session into another call session, determine the status of an individual call participant or a complete call session, and finally to end a call session.

### 8.1.1	Operation: MakeCallSession

The invocation of MakeCallSession requests to set-up a call session between two addresses, a first CallParticipant (“A-Party”) and a second CallParticipant (“B-Party”), provided that the invoking application is allowed to connect them. Optionally, the caller’s (“A-Party’s”) name is provided. Optionally a call session with only a single CallParticipant (“A-party”) can be set-up, for example to play media to the call participant using mediaCall capabilities (e.g. playMedia).Optionally the application can also indicate the charging information (Charging). Optionally, the media type(s) to be used for the participants in the call session can be requested (MediaInfo). A service policy details if multimedia application control is supported. If the parameter MediaInfo is omitted, the media type(s) shall be negotiated by the underlying network. A CallSessionIdentifier is returned on invocation of the MakeCallSession. This CallSessionIdentifier may be used to retrieve the call session status for all the call participants including their media type information with the operation getCallSessionInformation. With the operation getCallParticipantInformation this information may be retrieved for a single call participant.

During call set-up, the first CallParticipant (“A-Party”) might wait for the second CallParticipant (“B-Party”) to answer the call hearing ringback tone. MakeCallSession creates a call session with one or two call participants and generates a new CallSessionIdentifier which is identified by the string returned in the MakeCallSessionResponse operation. The invocation returns as soon as the request is received by the system, i.e. the actual call session is established asynchronously.

In order to receive the information on call status the application has to explicitly invoke the GetCallSessionInformation operation for the CallSessionIdentifier returned.

#### 8.1.1.1	Input message: MakeCallSessionRequest

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| CallParticipants | xsd:anyURI[1..2] | No | It contains the address of the first participant, and if supplied the second participant, involved in the call session. |
| CallingParticipantName | xsd:string | Yes | It contains the name of the caller, e.g. the name on whose behalf the call session is being established. |
| Charging | common:ChargingInformation | Yes | Charge to apply to the call session |
| MediaInfo | common: MediaInfo [0..unbounded] | Yes | It identifies one or more media type(s) for the call, i.e. the media type(s) to be applied to the participants in the call session. For each media type the media direction: incoming, outgoing, or bidirectional shall be indicated. An empty array shall have the same meaning as if the parameter is omitted. If the parameter MediaInfo is omitted, the media type(s) shall be negotiated by the underlying network. |
| ChangeMediaNotAllowed | xsd: boolean | No | If true, no call participant (user) in the call will be permitted to change media type during the call. If false the end user may change media type after the call is established as no network protection mechanism is set up to prevent participant (end user) initiated change of media type. |

#### 8.1.1.2	Output message : MakeCallSessionResponse

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| CallSessionIdentifier | xsd:string | No | It identifies the specific call session created |

#### 8.1.1.3	Referenced faults

ServiceException from 3GPP TS 29.199-1 [6]:

- SVC0001 - Service error.

- SVC0002 - Invalid input value.

PolicyException from 3GPP TS 29.199-1 [6]:

- POL0001 - Policy error.

- POL0008 - Charging not supported.

- POL0011 – Media type not supported

### 8.1.1a	Operation: AddCallParticipant

The invocation of AddCallParticipant requests to add a call participant to an existing call session.
The AddCallParticipant operation takes two mandatory parameters, the first being the call session identifier to which the participants should be added, and the second being the participant to add (contains the URI of the participant). Optionally, the media type(s) that shall be added for the specific call participant may be requested using MediaInfo. 
If the parameter MediaInfo is omitted, the media type(s) shall be negotiated by the underlying network. The call session or call participant status, including information on media types, can be retrieved using the operations getCallParticipantInformation, and getCallSessionInformation.

All occurrences of invalid call session or participant address shall result in an invalid input value exception.

#### 8.1.1a.1	Input Message : AddCallParticipantRequest

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| CallSessionIdentifier | xsd:string | No | It identifies the existing call session. This must be a non-null value as it identifies a pre-existing call session in the network |
| CallParticipant | xsd:anyURI | No | It contains the address of the user to add to the existing call session identified by the CallSessionIdentifier |
| MediaInfo | common: MediaInfo [0..unbounded] | Yes | It identifies one or more media type(s) for the participant to be added in the call session. For each media type the media direction: incoming, outgoing, or bidirectional shall be indicated. An empty array shall have the same meaning as if the parameter is omitted.If the parameter MediaInfo is omitted, the media type(s) shall be negotiated by the underlying network. |

#### 8.1.1a.2	Output message : AddCallParticipantResponse

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| None |  |  |  |

#### 8.1.1a.3	Referenced faults

ServiceException from 3GPP TS 29.199-1 [6]:

- SVC0001 - Service error.

- SVC0002 - Invalid input value.

- SVC0261 - Call Session already terminated.

PolicyException from 3GPP TS 29.199-1 [6]:

- POL0001 - Policy error.

- POL0011 – Media type not supported

- POL0240 – Too many participants. - from 3GPP TS 29.199-12 [7]

### 8.1.1b	Operation: TransferCallParticipant

The invocation of TransferCallParticipant enables a call transfer, effectively transferring a call participant from one call session into another call session.

The TransferCallParticipant method takes three parameters, the first being the call session  identifier (destination call session) where the participants should be moved to and the second being the source call session where the participant to transfer is currently located. The third parameter is to identify the call participant to transfer (contains the URL of the participant). Upon completion of the TransferCallParticipant operation, the call participant is implicitly deleted from the source call session.

In transferring the call participant to the new destination call session, the participant information is not affected. When, as a result of transferring a call participant, the source call session is left without any call participant, the source call session is terminated.

All occurrences of invalid call session or participant address shall result in an invalid input value exception

#### 8.1.1b.1	Input Message : TransferCallParticipantRequest

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| DestinationCallSessionIdentifier | xsd:string | No | It identifies the existing target call session to which the call participant to transfer shall be moved. |
| SourceCallSessionIdentifier | xsd:string | No | It identifies the existing source call session, which contains the call participant to transfer. |
| CallParticipant | xsd:anyURI | No | This is the address of the participant to be transferred from the source call session, which is identified by the SourceCallSessionIdentifier. |

#### 8.1.1b.2	Output message : TransferCallParticipantResponse

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| None |  |  |  |

#### 8.1.1b.3	Referenced faults

ServiceException from 3GPP TS 29.199-1 [6]:

- SVC0001 - Service error.

- SVC0002 - Invalid input value.

- SVC0261 - Call Session already terminated.

PolicyException from 3GPP TS 29.199-1 [6]:

- POL0001 - Policy error.

- POL0240 – Too many participants. - from 3GPP TS 29.199-12 [7]

### 8.1.2	Operation: GetCallParticipantInformation

The invocation of GetCallParticipantInformation retrieves the current call participant status, CallParticipantInformation, of the call participant identified by CallParticipant, within the call session identified by the CallSessionIdentifier.  This method can be invoked multiple times by the application even if the call has already ended. However, after the call has ended, status information will be available only for a limited period of time that is specified in the service policy 'StatusRetentionTime'.

The getCallParticipantInformation method takes two parameters into the method, the first being the call session  identifier where the participants information should be retrieved and the second being the call participant identifier (contains the URL of the participant).

All occurrences of invalid call session or call participant address shall result in an invalid input value exception

#### 8.1.2.1	Input message: GetCallParticipantInformationRequest

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| CallSessionIdentifier | xsd: string | No | It identifies the specific call session |
| CallParticipant | xsd:anyURI | No | It identifies a specific call participant address within the call session |

#### 8.1.2.2	Output message : GetCallParticipantInformationResponse

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| result | common: CallParticipantInformation | No | It identifies the status of the requested call participant |

#### 8.1.2.3	Referenced faults

ServiceException from 3GPP TS 29.199-1 [6]:

- SVC0001 - Service error.

- SVC0002 - Invalid input value.

- SVC0261 - Call Session already terminated.

PolicyException from 3GPP TS 29.199-1 [6]:

- POL0001 - Policy error.

### 8.1.3	Void

### 8.1.4	Void

### 8.1.5	Operation: GetCallSessionInformation

The invocation of GetCallSessionInformation retrieves the CallSession information regarding the call session identified by CallSessionIdentifier parameter.  The information provided in the CallSession includes the call participant information. This method can be invoked multiple times by the application even if the session has already ended. However, after the session has ended, status information will be available only for a limited period of time that is specified in the service policy 'StatusRetentionTime'.

All occurrences of invalid call session shall result in an invalid input value exception

#### 8.1.5.1	Input message: GetCallSessionInformationRequest

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| CallSessionIdentifier | xsd: string | No | It identifies a specific call session |

#### 8.1.5.2	Output message : GetCallSessionInformationResponse

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| result | common: CallSession | No | It identifies the status of the session |

#### 8.1.5.3	Referenced faults

ServiceException from 3GPP TS 29.199-1 [6]:

- SVC0001 - Service error.

- SVC0002 - Invalid input value.

- SVC0261 - Call Session already terminated.

PolicyException from 3GPP TS 29.199-1 [6]:

- POL0001 - Policy error.

### 8.1.6	Operation: DeleteCallParticipant

The invocation of DeleteCallParticipant  removes the call participant identified by CallParticipant from the call session identified by CallSessionIdentifier, and implicitly terminates that participants involvement in the call session.

The deleteCallParticipant method takes two parameters, the first being the call session  identifier where the participant to delete is located. The second parameter identifies the call participant to be deleted (contains the URL of the participant).

All occurrences of invalid call session or participant address  shall result in an invalid input value exception

#### 8.1.6.1	Input message: DeleteCallParticipantRequest

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| CallSessionIdentifier | xsd: string | No | It identifies a specific call session. |
| CallParticipant | xsd: anyURI | No | It identifies a specific call participant within the identified call session. |

#### 8.1.6.2	Output message: DeleteCallParticipantResponse

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| None |  |  |  |

#### 8.1.6.3	Referenced faults

ServiceException from 3GPP TS 29.199-1 [6]:

- SVC0001 - Service error.

- SVC0002 - Invalid input value.

- SVC0261 - Call Session already terminated.

PolicyException from 3GPP TS 29.199-1 [6]:

- POL0001 - Policy error.

### 8.1.7	Operation: EndCallSession

The invocation of EndCallSession terminates the call session identified by CallSessionIdentifier.

The call to all participants is ended.

All occurrences of invalid call session shall result in an invalid input value exception

#### 8.1.7.1	Input message: EndCallSessionRequest

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| CallSessionIdentifier | xsd: string | No | It identifies a specific call session |

#### 8.1.7.2	Output message: EndCallSessionResponse

| Part name | Part type | Optional | Description |
| --- | --- | --- | --- |
| None |  |  |  |

#### 8.1.7.3	Referenced faults

ServiceException from 3GPP TS 29.199-1 [6]:

- SVC0001 - Service error.

- SVC0002 - Invalid input value.

- SVC0261 - Call session already terminated.

PolicyException from 3GPP TS 29.199-1 [6]:

- POL0001 - Policy error.

# 9	Fault definitions

The following faults are defined for this service.

## 9.1	ServiceException

### 9.1.1	SVC0260: Void

### 9.1.2	SVC0261: Call Session already terminated

| Part name | Description |
| --- | --- |
| Message Id | SVC0261 |
| Text | Call session has already been terminated |
| Variables | None |

# 10	Service policies

These service policies are defined for the Third Party Call service.

| Name | Type | Description |
| --- | --- | --- |
| ChargingAllowed | xsd:boolean | Indicates whether charging is allowed for makeCallSession operation |
| StatusRetentionTime | common:TimeMetric | Length of time to retain status after the termination of the call |
| ChangeMediaNotAllowed | xsd:boolean | Indicates whether an end user can change the media types used in a call. |
| MaximumParticipants | xsd:int | Maximum number of participants which a session can serve. |
| MultimediaSupported | xsd:boolean | Indicates whether multimedia is supported and whether an application can change the media types used in a call. |

Annex A (normative):
WSDL for Third Party Call

The document/literal WSDL representation of this interface specification is compliant to 3GPP TS 29.199-1 [6] and is contained in text files (contained in archive 29199-02-800-doclit.zip) which accompanies the present document.

Annex B (informative): 
Description of Parlay X Web Services Part 2: Third party call for 3GPP2 cdma2000 networks

This annex is intended to define the OSA Parlay X Web Services Stage 3 interface definitions and it provides the complete OSA specifications. It is an extension of OSA Parlay X Web Services specifications capabilities to enable operation in cdma2000 systems environment. They are in alignment with 3GPP2 Stage 1 requirements and Stage 2 architecture defined in:

[1]	3GPP2 X.S0011-D: “cdma2000 Wireless IP Network Standard ", Version 1.1

[2]	3GPP2 S.R0037-0:  "IP Network Architecture Model for cdma2000 Spread Spectrum Systems", Version 3.0

[3]	3GPP2 X.S0013-A: "All-IP Core Network Multimedia Domain"

These requirements are expressed as additions to and/or exclusions from the 3GPP Release 8 specification. 
The information given here is to be used by developers in 3GPP2 cdma2000 network architecture to interpret the 3GPP OSA specifications.

# B.1	General Exceptions

The terms 3GPP and UMTS are not applicable for the cdma2000 family of standards. Nevertheless these terms are used (3GPP TR 21.905) mostly in the broader sense of "3G Wireless System". If not stated otherwise there are no additions or exclusions required.

CAMEL mappings are not applicable for cdma2000 systems.

# B.2	Specific Exceptions

## B.2.1	Clause 1: Scope

There are no additions or exclusions.

## B.2.2	Clause 2: References

There are no additions or exclusions.

## B.2.3	Clause 3: Definitions and abbreviations

There are no additions or exclusions.

## B.2.4	Clause 4: Detailed service description

There are no additions or exclusions.

## B.2.5	Clause 5: Namespaces

There are no additions or exclusions.

## B.2.6	Clause 6: Sequence diagrams

There are no additions or exclusions.

## B.2.7	Clause 7: XML Schema data type definition

There are no additions or exclusions.

## B.2.8	Clause 8: Web Service interface definition

There are no additions or exclusions.

## B.2.9	Clause 9: Fault definitions

There are no additions or exclusions.

## B.2.10	Clause 10: Service policies

There are no additions or exclusions.

## B.2.11	Annex A (normative):	WSDL for Third Party Call

There are no additions or exclusions.

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Dec 2005 | CT_30 | CP-050582 | 0003 | -- | Modify TPC to allow playing announcement | C | 6.2.0 | 7.0.0 |
| Jun 2006 | CT_32 | CP-060208 | 0006 | -- | Apply Union data type element naming convention | F | 7.0.0 | 7.1.0 |
| Dec 2006 | CT_34 | CP-060598 | 0007 | -- | Add Extended Call Control for Parlay X | C | 7.1.0 | 7.2.0 |
| Mar 2007 | CT_35 | CP-070045 | 0009 | -- | Add OSA Parlay Web Services support for 3GPP2 networks | A | 7.2.0 | 7.3.0 |
| Mar 2007 | CT_35 | CP-070048 | 0010 | -- | Add support for multimedia in Third party call | B | 7.2.0 | 7.3.0 |
| Mar 2007 | -- | -- | -- | -- | Editorial: Aligned 5 Namespaces | -- | 7.3.0 | 7.3.1 |
| Jun 2007 | CT_36 | CP-070346 | 0012 | -- | Correction to Third Party Call Web Service | F | 7.3.1 | 7.4.0 |
| Jun 2007 | CT_36 | CP-070346 | 0013 | -- | Refine Third Party Call Web Service | F | 7.3.1 | 7.4.0 |
| Sep 2007 | CT_37 | CP-070641 | 0014 | -- | Add service scenario to Third Party Call web service | C | 7.4.0 | 8.0.0 |