Request Connection

|  | Request | Reply |  |  |
| --- | --- | --- | --- | --- |
|  |  | Confirm | Reject |  |
| Req ID | RA | RA | RA |  |
| CID | Null | PA | Null |  |
| STP-A | EdgePoint | STP | EdgePoint | reason |
| -- STP-A attr |  |  |  |  |
| STP-B | EdgePoint | STP | EdgePoint | reason |
| Start Time | Range | T1 | Range | reason |
| End Time | Range | T2 | Range | reason |
| Service attr | Range | SI | Range | reason |
| Path attr | Req PP | CO seq | Req PP | reason |

Table entries in Bold provided by the NSA in that column

Table entries in italics are copied from the entry in bold

Table shows set of fields in NSI messages between Requestor and Provider NSAs.

Reply is from provider and may be either a confirmed connection or a reject.

If the reply is a confirm it describes a Connection object.  The fields describe everything known about the connection.

If the reply is a reject, the fields duplicate what was in the request, and at least one field has a reason code that defines a request field that could not be satisfied by the provider.

Request ID – Identifier of the particular request.  Used to match requests and replies.  RA is a Requestor Assigned identifier.

CID –  The Connection Identifier is assigned by the provider and is an identifier unique to the provider.

STP-A/B is an edge of a particular connection.  In the request the field is defined as an EdgePoint, where an edge point might be able to provide multiple STPs.  An EdgePoint might be a single STP.

In the Confirm response, the STP field is a single STP.

STP attr – is the characteristics of the STP.  In the request it might be a range of STPs at a EdgePoint .  In all messages it will be the type of STP – e.g. VLAN or WAVE.  Other attributes of STPs may be defined in the future.

Start and End Time – In a request these may be a range, while in a reply the Start Time is the  beginning of when resources are available to make the connection and end time is the time when resources are no longer available.  [a question – do we want to include attributes of times, e.g. “may be preempted after 20 minutes”]

Service attr – these are attributes that describe a Service Instance (based on a Service Definition).  The requested Service Instance defines a set of acceptable parameters.  The Service attributes in a confirmed reply are the specific values of service parameters of the connection.  Service attributes define performance attributes of the connection.

Path attr – This is an optional field.  In the request message it defines path attributes that must be satisfied by the provider.  Path attributes in a request might include EdgePoints, Networks,  or other Topological elements that are defined.

In a  confirm reply the path attributes are Connection Objects that are segments provided by child NSAs.  The presence of Path attributes in the confirm reply is optional from a NSI protocol definition;  a particular federation of networks may require this .

ProvisionConnection

These messages use Connection Objects from Provider.  The requestor provides a new ReqID.

Note that ProvisionConnection sequence is not needed if Automatic provisioning is specified in Connection Reservation

Cancel Connection

This message technically only needs the CID.  The requestor provides a new ReqID.