G-lambda Interface Parameters

# Introduction

The objective of this document is to identify parameters for the G-lambda interface (GNS-WSI3: Grid Network Service – Web Services Interface version 3).

In this document, parameters are classified into six categories.

- Path: fundamental path parameters including endpoint

- Bandwidth:

- Time: time parameters for advance reservation

- Path properties: Additional path parameters (e.g. QoS, VLAN)

- Network Service Agent properties: NSA information related to SLA and so on.

- Others:

# Path

Path parameters include endpoint and route information.  Each endpoint includes domain and termination point (physical interface) information.  Physical interface is specified by media attachment units [IANA-MAU-MIB].  Route information has subpaths, which may be managed by the other network service agents.

Table1. Path parameters

| Entity | Inherits from |  | Description |  |
| --- | --- | --- | --- | --- |
| Path |  |  | A pair of logical network endpoints. When a path allocates over different domains, a path may contain sub-paths. |  |
| Property | Type | Multi. |  | Description |
| APoint | Endpoint | 1 |  | An endpoint in a path. An APoint must be paired with a ZPoint. |
| ZPoint | Endpoint | 1 |  | An endpoint in a path. A ZPoint must be paired with an APoint. |
| Route | Route | 0..1 |  | A list of subsets of the path. |
| Bandwidth | Bandwidth | 1 |  | An abstracted element of a requesting and assigned bandwidth. |
| PathProperties | PathProperties | 0..1 |  | A set of path information. |

Table2. Endpoint parameters

| Entity | Inherits from |  | Description |  |
| --- | --- | --- | --- | --- |
| Endpoint |  |  | A termination site of a path or sub-path. An endpoint may contain termination points of connections. |  |
| Property | Type | Multi. |  | Description |
| DomainName | string | 0..1 |  | A name of a domain, which is a network administrated by a telecommunication carrier, internet service provider, organization, group, or some other type of authority. |
| TerminationPoint | TerminationPoint | 0..* |  | A physical or logical termination point of a connection. |

Table3. Termination point parameters

| Entity | Inherits from |  | Description |  |
| --- | --- | --- | --- | --- |
| TerminationPoint |  |  | A physical or logical termination point of a connection. |  |
| Property | Type | Multi. |  | Description |
| TerminationPointName | string | 0..1 |  | A name of a termination point. |
| MediaType | MediaType | 0..1 |  | A media type of a termination point. |

Table4. Subpath parameters

| Entity | Inherits from |  | Description |  |
| --- | --- | --- | --- | --- |
| SubPath |  |  | A pair of logical network endpoints. An NRM handling a sub-path may be different domain from the domain of the NRM handling a path. |  |
| Property | Type | Multi. |  | Description |
| APoint | Endpoint | 1 |  | An endpoint in a path. An APoint must be paired with a ZPoint. |
| ZPoint | Endpoint | 1 |  | An endpoint in a path. A ZPoint must be paired with an APoint. |
| Nrm | Nrm | 1 |  | An NRM information handling a sub-path. |

# Bandwidth

Requestor agents request a bandwidth with “MinimunBW”, “MaximumBW” and “PeakBW” scheme.  According to the request, network service agents decide an assigned bandwidth, and reply with “GuaranteedBW” and “PeakBW scheme.

To describe a bandwidth (named “GeneralBW”), G-lambda interface uses a bandwidth value and a bandwidth unit.

Table5 A bandwidth parameters

| Entity | Inherits from |  | Description |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | A bandwidth. |  |
| Property | Type | Multi. |  | Description |
| MinimumBW | GeneralBW | 1 |  | An acceptable minimum bandwidth requesting from a requestor agent to be guaranteed. The guaranteed bandwidth must not be assigned below the value of this parameter. When a MaximumBW is given, a network service agent decides the assigned bandwidth between a MinimumBW and a MaximumBW. |
| MaximumBW | GeneralBW | 0..1 |  | An acceptable maximum bandwidth requesting from a requestor agent to be guaranteed. In a commercial network, clients can pay within the value of this parameter. A network service agent decides the assigned bandwidth between a MinimumBW and a MaximumBW. This parameter must be requested with a MinimumBW. |
| PeakBW | GeneralBW | 0..1 |  | A requesting bandwidth without guarantee. The value of this parameter should be equal to or less than the rate of the physical network interface. |
| GuaranteedBW | GeneralBW | 1 |  | An assigned bandwidth with guarantee. |

# Time

Requestor agents can request reservation time with two different schemes.  With “Exact” scheme, RAs designate specific start/end times.  With “Range” scheme, RAs designate desired start/end times and specific reservation duration.  Duration parameters are constructed from a value and unit.

Table6. An exact time parameters

| Entity | Inherits from |  | Description |  |
| --- | --- | --- | --- | --- |
| Exact |  |  | Exact time frame type |  |
| Property | Type | Multi. |  | Description |
| StartTime | xsd:dateTime | 1 |  | Requesting/assigned time when a reservation will be activated. |
| EndTime | xsd:dateTime | 1 |  | Requesting/assigned time when a reservation will be deactivated. |

Table7. A range time parameters

| Entity | Inherits from |  | Description |  |
| --- | --- | --- | --- | --- |
| Range |  |  | Ranged time frame type |  |
| Property | Type | Multi. |  | Description |
| Duration | Duration | 1 |  | Requesting period of time to reserve. |
| EarliestStartTime | xsd:dateTime | 0..1 |  | Earliest requesting time when a reservation will be activated. |
| LatestStartTime | xsd:dateTime | 0..1 |  | Latest requesting time when a reservation will be deactivated. |

# Path properties

Path properties are additional parameters, when a requestor agent wants to designate QoS or switching schemes.  QoS properties of different layers can be included in a path.

## General properties

- Availability: The ratio of the total time that a path is capable of being used in a year.

- TransmissionDelay: The acceptable one-way time to transfer between an APoint and a ZPoint. This parameter doesn't include a forwarding delay in switches or routers.

- SwitchingScheme: A switching/routing scheme in a path.

## Ethernet properties

- MTU: A requested/allowable MTU.

- VLAN Tag ID: A parameter for configuring virtual LANs in the IEEE 802.1Q header.

- CoS: A parameter to differentiate traffic in the Ethernet frame header.

## MPLS properties

- LSP ID: A parameter to create a label switched path (LSP) in MPLS networking.

- EXP: A parameter to differentiate traffic in the MPLS header.

## IP properties

- PacketLossRate: The ratio of total lost IP packet outcomes to total transmitted IP packets in a population of interest. The detailed definition is described at [Y.1540] as IPLR (IP Loss Rate).

- PacketErrorRate: The ratio of total errored IP packet outcomes to the total of successful IP packet transfer outcomes plus errored IP packet outcomes in a population of intrest. The detailed definition is described at [Y.1540] as IPER (IP Error Rate).

- AverageJitter: The average variation of the end-to-end delay from a packet to the next packet.

- MaximumJitter: The maximum variation of the end-to-end delay from a packet to the next packet.

- Latency: The average one-way IP packet transfer time for a population of interest between an APoint and a ZPoint with forwarding delays at switches and routers. The detailed definition is described at [Y.1540] as IPTD (IP Transfer Delay).

- LatencyVariation: The variation of a one-way latency within a stream of packets. The detailed definition is described at [Y.1540] as IPDV (IP Delay Variation).

- PacketReordering: NRM provides the guarantee to transfer packets in order or not.

- ToS: A parameter reserved for the service type in the IPv4 header.

# Network Service Agent properties

Network Service Agent information.

- SLAAvailability: The ratio of the total time that an NRM is capable of being used in a year.

- ProvisioningDelay: The maximum time to activate an assigned path.

- FailureReportDelay: The maximum time to notify clients of failures.

- FailureRepairTime: The maximum time to recover a path from out of service.

- Redundancy: The configuration to ensure the path continues under the failure(s).

# Others

For authentication, the identifier of requestor agents is required.