## Transport object terms

| Term | Sub classifications | Definition |
| --- | --- | --- |
| Link |  | NML term – used as defined in NML. Describes a transport layer infrastructure resource that connects service termination points (STP). |
| Node |  | NML term – used as defined in NML. Describes a transport layer infrastructure resource that is typically one or more units of network equipment – such as an Ethernet switch |
| Connection |  | This is a service layer term that identifies a group of transport plane resources that may be reserved to create an ETE connection. The concept of connection maps directly to the NML concept of a link (is this correct?) |
|  | Network connection | This distinguishes a sub category of connections that connect two STPs in a single network. |
|  | ETE connection | This distinguishes a sub category of connections that connect two user STPs – note that the hierarchical concept in NSI means that the user connection points are an ill-defined concept. |
| Segment |  | The Segment concept is used to help describe the process of concatenation. Essentially a segment is the same as a connection. A connection is made up of concatenated segments. |
| Port |  | NML term. A port is a point where two segments may be joined. In NML definition a port in on a node. In NSI case port is a point that identifies the end of a connection. It has been suggested that Ports should be known as Service Termination Points (STPs) to avoid confusion between physical ports and STP which may be physical or virtual. Several sub-classifications of the port concept have been identified in discussions. -End ports vs. intermediate ports -Physical ports vs. virtual ports |
|  | Link port | A port which terminates a link |
|  | Access port | The port at which a user connects |
|  | Forwarding port | ? |
|  | Physical port | Physical port – eg an Ethernet trunk port |
|  | Virtual port | Virtual port – eg a vlan |
| Service Termination point |  | Alternative for virtual access port – Chin suggests calling this calling this Service Termination Point (STP) |
| Network |  | Describes a group of objects in a transport plane that are managed by a single NSA. |
| Domain |  | ? |
| Routing area |  | Networks can be aggregated into routing areas. These are areas which have full reachability. If an end address is outside of routing area, the area will have a forwarding table to appropriate routing area. |

## Plane terms

| Term | Sub classifications | Definition |
| --- | --- | --- |
| Service plane |  | The service plane is a new concept and is defined in this document to refer to the plane that contains a set of Network Service Agents and Network Service interfaces instances. The service plane provides a set of systems and processes that are geared towards providing services to users and maintaining state on those services. The Service plane will generally rely on the functions of the control plane and/or management plane to effect actual changes on the transport plane. In addition, the service plane will typically maintain databases on current and future service instantiations and coordinate associated workflow processes. |
| Control plane |  | Control plane is used in accordance with the ITU-T G.8080. The control plane incorporates the set of control plane components that are used to manipulate transport network resources in order to provide the functionality of setting up, maintaining and releasing connections. |
| Management plane |  | Management plane is used in accordance with the ITU-T M.3010 TMN definition: The management layer contains the management functions that are responsible for the management, including coordination of activity, of a network view. |
| Transport plane |  | Transport plane is used in accordance with the ITU-T G.800 definition: The transport plane is the plane that contains transport functions which transfers any telecommunications information from one point to another point(s). |

## NSA terms

| Term | Sub classifications | Definition |
| --- | --- | --- |
| NSA |  | Network Service Actor JA -An actor is a software entity that, in response to a NSI message, can concurrently initiate any internal processes to serve the request; send a finite number of messages to other actors basing on internal service policies; and designate the behavior to be used for the next message it receives |
|  | NSA Requestor | An NSA which originates a connection request |
|  | NSA Provider | An NSA which terminates and processes a connection requests. |
| NSI |  | Network Service Interface |
|  | provider-to-provider instance of NSI | No formal definition required. |
|  | requestor-to-provider instance of NSI | No formal definition required. |
| NSI Agent |  | The agent that terminates the NSI interface and implements sending and receiving of NSI protocol messages. Agent for terminating NSI the word "agent" (formerly "software agent") is used for naming a piece of software that acts for a user or another program in the relationship of agency. |
|  | Provider Agent | Source end of NSI request |
|  | Requestor Agent | Sink end of NSI request |
| RMA |  | Resource Manager Agent |
| Network information services |  | Service which provides |
| Topology service |  | A service used to exchange the topologies that are provided by and used by NSAs |
| Agent resolution service |  | A service used to resolve the URL of an NSA |
| Path computation service |  | A service used to return a set of paths based on path computation. |
| Scheduling service |  | A calendar service used to record the state of reservations with time information |
| Segment aggregation service |  | A service for splitting connections out into segments and for aggregating multiple connection responses. |
| Service termination point registration |  | A service for looking up the details of a customer port. |

## Connection request terms

| Term | Sub classifications | Definition |
| --- | --- | --- |
| path |  | A full or partial list of path end points and segments which describe the route of a connection. The path information is provided by the path computation service. |
| Bandwidth |  | The bandwidth requested by a user and included in the connection request. |
| Connection id |  | Globally unique identifier for describing a connection request. |